# Computer Mouse – Internal Circuit Description

## Introduction

A computer mouse is an input device used to control the movement of the cursor on a computer screen. The image shows the internal printed circuit board (PCB) of an optical USB mouse. This circuit controls movement detection, button clicks, and communication with the computer.

## Main Components Identified in the Picture

### 1. Optical Sensor (Center IC)

- Located at the center of the PCB.

- Detects movement by tracking changes on the surface below the mouse.

- Works together with the LED light source.

- Converts motion into digital signals.

### 2. LED (Light Emitting Diode)

- Positioned near the optical sensor.

- Illuminates the surface beneath the mouse.

- Helps the sensor accurately detect movement.

- Marked with polarity (+) on the PCB.

### 3. Microcontroller / Controller IC

- Processes signals from the optical sensor.

- Converts movement and button clicks into data.

- Sends data to the computer through the USB cable.

### 4. Left and Right Click Switches

- Mechanical switches located at the bottom left and right.

- Register left-click and right-click actions.

- Each press completes a circuit.

### 5. Middle Button (Scroll Button)

- Used for scrolling and middle-click actions.

- Located between the left and right buttons.

### 6. USB Cable Connections

- Wires visible on the side of the PCB.

- Typical colors:

  - Red: Power (+5V)

  - Black: Ground

  - Green: Data+

  - White: Data–

- Connects the mouse to the computer.

### 7. Resistors and Capacitors

- Control current flow and voltage stability.

- Ensure smooth and reliable operation of the mouse.

## Working Principle

- The LED lights up the surface below the mouse.

- The optical sensor captures surface movement.

- The controller processes the movement data.

- Button presses are detected by switches.

- Data is sent to the computer via USB.

- The cursor moves and clicks are registered on the screen.

### ┌──────────┐

### │  START   │

### └──────────┘

### ↓

### ┌──────────────────┐

### │ USB Power Supply │

### │   (5V from PC)   │

### └──────────────────┘

### ↓

### ┌──────────────────┐

### │ LED Emits Light  │

### │ (Illuminates     │

### │ surface)         │

### └──────────────────┘

### ↓

### ┌──────────────────┐

### │ Optical Sensor   │

### │ Detects movement │

### └──────────────────┘

### ↓

### ┌──────────────────┐

### │ Microcontroller  │

### │ Processes data   │

### └──────────────────┘

### ↓

### ┌──────────────────────────┐

### │ Button / Scroll Input ?  │

### └──────────────────────────┘

### ↓

### ┌──────────────────┐

### │ Data sent to PC  │

### │ via USB cable    │

### └──────────────────┘

### ↓

### ┌──────────────────┐

### │ Cursor Movement  │

### │ on Screen        │

### └──────────────────┘

### ↓